RUGTEL

RT 200 – User’s manual

FEATURES

● Caller ID

● ⁠Redial

● ⁠Music on hold

● ⁠DTMF/FSK compatible

● ⁠LCD contrast adjustable

● ⁠Tone/Pulse selectable

● ⁠Flash

● ⁠Call log

● ⁠ Desk/Wall mountable

Setting Menu

In on hook state, press FLASH/SET key to enter setting mode, now the LCD displays SET 12345678, press the numerical key or UP key or DOWN key to select the setting item, the LCD will displays the relevant setting item. After finishing the setting, press DEL key back to the list, press the numerical key again or press UP key DOWN key to select next setting item.

Setting Year, Month, Date & Time

In on hook state, press FLASH/SET key, the LCD displays SET 12345678, press number key 1, LCD display 1—DATE、2—CL, press number key 1 to set Year, Month & Date and flashes, press digit keys directly to set the Year, Month & Date and press FLASH/SET key to confirm.

Press number key 2 to set Time setting, LCD display show ‘’CL 00-00 and 00-00 flashes, press digit keys directly to set time and press FLASH/SET key to confirm.

Setting Flash, Tone/Pulse functions:

In on hook state, press FLASH/SET key, LCD display show "SET 12345678", then press numeric key 5, LCD display show 1-f 2-pt 3-d，press numeric key 1 to set Flash timing, LCD display show flash 600, press UP or DOWN key to select the flash timing, then press FLASH/SET key to confirm.

Press numeric key 2 to set P-T, LCD display show pt tone, press UP or DOWN key to select the Tone or Pulse, then press FLASH/SET key to confirm.

Setting Local Area Code & PBX Code functions

In on hook state, press FLASH/SET key, LCD display show "SET 12345678", then press numeric key 6, LCD display show 1－LOC   2-OL, press numeric key 1 to set Local area code, LCD display show loc……,press digit keys directly to set local area code (up to 8 digits),then press FLASH/SET key to confirm. To cancel the local code, press the DEL key while entering the state to delete the entered number.

Press DEL key to return back, LCD display show 1－LOC   2-OL, press number key 2 to set PBX, LCD display show out off，press UP or DOWN key to ON/OFF the PBX, the setting method the same as setting local area code.

Setting LCD contrast

In on hook state, press FLASH/SET key, LCD display show "SET 1234567 8", then press numeric key 7, LCD display show LCD con 4, press UP or DOWN key to select LCD contrast. press FLASH/SET key to confirm.

Phone Operations

BACK

Press the back key directly to dial the selected number during pre-dial or number lookup; if you want to add "0" before the number, you need to press and hold the call back key for 3 seconds, you need to lift the handset immediately, and then you can dial out the number.

Review Incoming number

In the on-hook or off-hook state, press the up or down key to view the incoming call number; in the on-hook state, press and hold the # key for 3 seconds to quickly enter the incoming number state.

Review Outgoing call

In the on-hook or off-hook state, press the OUT key to view the outgoing call number, and then press the up or down key to view the outgoing call information; in the on-hook state, press and hold the * key for 3 seconds to quickly enter the outgoing call number state.

Pre-dialing

In the on-hook state, directly enter the phone number. If there is an error, you can use the DEL key to modify it. After the number is entered, you can pick up the phone or press the BACK button to dial the number directly. Press and hold the BACK for 3 seconds to add "0" to the callback, you need to lift the handset immediately to be able to dial the number.

Delete function

In the state of number input, you can use the DEL key to delete the wrong number; in the state of review numbers, you can use the DEL key to delete the current number and its information, or long press the DEL key for 3 seconds to delete all incoming or outgoing numbers.

Music on hold

In off-hook state, press the hold key to enter the music on-hold state, and the on-hold music sounds at the same time, at this time, you can put down the handset, off-hook again, the on-hold music stops, and automatically enter the handset call state.

GENERAL INFORMATION

1. For your own safety, avoid using the phone in place with excessive heat and humidity.

2. Position the phone at least one meter away from equipment such as, cordless phones or TV. These equipments may cause interference in the phone.

3. At the end of the phone's life, recycle it in a way that it does not harm the environment, in accordance with local regulations.

4. In order to recycle/dispose the product please visit www.eprewastecpcb.in.

WARRANTY

This Rugtel push button telephone set is warranted for a period of 13 months from the date of purchase. Any defective instrument will be repaired by Reliable Comnet Pvt. Ltd.(the company) on back to bench basis under warranty period considering the following conditions:-

- Invoice copy should be presented to claim the warranty.

- On repair of the instrument the said warranty period thereafter remains unchanged to the original warranty.

- This warranty is not valid for:

- Damage caused by accident, mishandling, negligence, tampering, unauthorized repair, failure to follow instructions, act of God etc…

- Instrument purchased from dealer not registered by the company.

- This warranty excludes every claim not mentioned herein.

- In no event shall “Reliable Comnet Pvt. Ltd.” be liable for any direct, indirect, special, incidental or consequential damages whether based on warranty or any other legal theory due to delay in services.

- The company will not accept any responsibility for damage or loss incurred during transit of the telephone set/sets.

- Any claim to this warranty shall be only made before the courts having jurisdiction in Noida, Gautam Budh Nagar, 201301 (U.P.)

- Please note warranty is not applicable if sold outside India.

Customer care cell:

Reliable Comnet Pvt. Ltd.

G 225, Sec 63, Noida(201301)

E-mail ID: info@rcpl.in

Customer care number: 0120-4647100